Joule: Dynamic Power Envelope Control
for Power-Constrained Compute Clusters

Finn Wicks

March 2026

Abstract	3

1. Problem	3

2. Observation	4

3. Approach	4

3.1 Control Loop	4

3.2 Marginal Power Allocation	5

4. Architecture	6

4.1 Cluster Controller	6

4.2 Node Agents	6

4.3 Hardware Power Interfaces	6

4.4 Node Telemetry	7

4.5 Power Limit Enforcement	7

4.6 Fault Tolerance	7

4.7 Implementation	8

5. Experimental Methodology	8

5.1 Hardware Platform	8

5.2 Workloads	8

5.3 Experimental Procedure	8

5.4 Cap Sweep	8

5.5 Metrics	9

5.6 Control Plane Overhead	9

5.7 Reproducibility	9

6. Results	10

6.1 Power Tracking	10

6.2 Cap utilisation	10

6.3 Throughput Improvement	10

6.4 Cap Error Distribution	10

6.5 Overshoot	10

7. Implications	10

8. Limitations	10

9. Future Work	11

10. Conclusion	12

Abstract

Modern compute systems increasingly operate under strict power constraints. datacentres, racks, and individual servers frequently enforce power caps to maintain thermal limits, power delivery budgets, or facility-level constraints. However, static power allocations often leave significant power headroom unused due to workload variability and conservative configuration. This unused power represents lost computational capacity.

This work introduces Joule, a power-aware control plane that dynamically allocates power budgets across nodes while maintaining strict compliance with a global power envelope. Joule continuously measures node-level power consumption and workload throughput, using a feedback control loop to adjust per-node power limits. The system aims to maximize useful work while ensuring that total cluster power remains within the specified cap.

We evaluate Joule using sustained workload experiments under multiple power caps. Results demonstrate that Joule significantly improves power-cap utilisation and increases aggregate throughput without violating the global power envelope.

Traditional cluster schedulers treat compute resources as primary and power as a secondary constraint. Joule inverts this relationship by treating power as the primary resource and allocating compute work within the available electrical envelope.

In our evaluation, Joule improves power-cap utilisation from roughly 80–85% under static configurations to more than 98% while maintaining zero cap violations.

1. Problem

Power has become one of the primary constraints in modern computing infrastructure. Datacentres operate within strict limits imposed by power delivery systems, cooling capacity, and facility-level electrical budgets. These constraints are often enforced through power caps applied at the server or rack level.

In practice, power caps are typically configured statically. Administrators assign fixed limits to nodes or workloads to ensure the overall system remains within safe operating bounds. While this approach is simple, it introduces a fundamental inefficiency: workloads rarely consume their allocated power budget continuously.

Application behaviour fluctuates over time due to changes in instruction mix, memory pressure, or I/O activity. As a result, instantaneous power consumption often falls well below the configured cap. When this occurs, available power headroom remains unused even though other workloads could potentially benefit from it.

This inefficiency is particularly costly in environments where compute throughput is tightly coupled to power availability. In such systems, unused power directly translates to lost computational capacity.

A more efficient system would dynamically redistribute power headroom across nodes in response to workload behaviour while maintaining strict adherence to the global power constraint.

2. Observation

Two observations motivate the design of Joule.

First, real workloads exhibit significant temporal variability in power consumption. Even highly optimized compute tasks rarely sustain maximum power draw continuously. Short-term fluctuations arise from pipeline stalls, memory latency, instruction mix variation, and operating system activity.

Second, modern processors expose mechanisms for fine-grained power control, such as hardware power limits that can be adjusted dynamically at runtime. These interfaces make it possible to modulate power consumption without interrupting running workloads.

Together, these observations suggest that a control system could continuously monitor power usage and dynamically adjust power limits in order to keep the system operating near the desired envelope.

Instead of assigning static limits to each node, a centralized controller can treat the cluster as a shared power budget and allocate watts where they produce the most useful work.

3. Approach

Joule implements this idea through a feedback-driven power allocation system.

The system maintains a global power cap representing the maximum allowable power draw for the cluster. Each node in the cluster receives a dynamic power limit that is adjusted continuously by the controller.

The core objective of the controller is to maximize useful work while ensuring that total cluster power remains within the configured cap.

The Joule control loop operates as follows:

- Each node periodically reports power consumption and workload throughput.

- The controller aggregates these measurements to determine current cluster power usage.

- If total power is below the configured cap, the controller increases node limits to utilize available headroom.

- If power approaches or exceeds the cap, the controller reduces limits to maintain compliance.

This feedback process enables the system to track the global power envelope closely while adapting to changing workload conditions.

To improve allocation decisions, Joule also estimates the marginal throughput benefit of additional power on each node. These estimates allow the controller to prioritize nodes that produce the greatest increase in work per additional watt.

3.1 Control Loop

Joule maintains a global power envelope using a feedback controller operating over the aggregate cluster power.

Let:

P_total(t) = measured total cluster power
P_cap      = configured power envelope
e(t)       = control error = P_cap − P_total(t)

The controller adjusts the total power budget allocated to nodes using a proportional–integral (PI) control law:

ΔL = Kp * e(t) + Ki * ∫ e(t) dt

Where:

- Kp controls immediate response to error

- Ki compensates steady-state tracking error

This formulation allows Joule to respond quickly to power fluctuations while maintaining stable long-term tracking of the envelope.

To ensure stability under delayed actuation and measurement noise, Joule incorporates several safeguards:

- deadband regions to avoid oscillation near the cap

- guard bands to prevent overshoot

- actuator slew limits

- emergency backoff when power exceeds safety thresholds

These mechanisms allow Joule to maintain tight power tracking without inducing unstable oscillatory behaviour.

To ensure strict compliance with the global power envelope, Joule intentionally maintains a small guard band below the configured cap. This margin accounts for measurement noise, actuation delay, and transient workload behaviour. By operating slightly below the cap, the controller prevents overshoot while still maintaining high utilisation of the available power budget.

3.2 Marginal Power Allocation

While the envelope controller determines the total power budget, Joule must decide how to distribute that budget across nodes.

To accomplish this, Joule estimates the marginal throughput benefit of additional power on each node.

Let:

H = throughput
P = power

Joule estimates marginal efficiency as:

m = dH / dP

Nodes with higher marginal throughput benefit receive a larger share of available power.

Because true derivatives are difficult to measure directly, Joule approximates these values using small probing adjustments and exponentially weighted moving averages.

This approach allows the system to adapt dynamically as workload behaviour changes.

4. Architecture

Joule is structured as a distributed control system consisting of three primary components:

- Cluster Controller ("Brain")

- Node Agents

- Hardware Power Interfaces

4.1 Cluster Controller

The central controller manages the global power envelope and determines the desired power allocation for each node. The controller periodically collects telemetry from all nodes and runs the control loop to update power limits.

The controller maintains safety mechanisms to ensure stable operation, including guard bands, overshoot protection, and stale-node detection.

4.2 Node Agents

Each node runs a lightweight agent responsible for local measurement and actuation. The agent performs several tasks:

- reading hardware power counters

- estimating instantaneous power usage

- measuring workload throughput

- applying power limits via hardware interfaces

Agents publish their current state to the controller and apply the desired limits computed by the control loop.

4.3 Hardware Power Interfaces

Power limits are enforced using processor power-management mechanisms exposed by the hardware platform. These interfaces allow software to adjust power limits dynamically while workloads continue running.

By combining centralized control with local measurement, Joule can adapt to workload changes while maintaining a coherent global power policy.

Figure 1 illustrates the high-level architecture of Joule, consisting of a centralized controller coordinating node agents that interact with hardware power interfaces.

Figure 1: Joule system architecture showing the centralized controller,
per-node agents, and hardware power control interfaces.

+----------------+

|  Joule Brain  |

+----------------+

/                         \

Node Agent A   Node Agent B

|                           |

Power                  Power

Sensors                 Sensors

|                            |

Hardware Power Interfaces

4.4 Node Telemetry

Each node runs a lightweight agent responsible for measuring power consumption and workload throughput.

Power measurements are derived from processor power counters and converted into instantaneous power estimates using energy-difference calculations over time intervals.

To reduce measurement noise, the agent maintains exponential moving averages of both power and throughput.

These smoothed values are reported periodically to the cluster controller.

4.5 Power Limit Enforcement

Power limits are applied using hardware power management interfaces exposed by the processor platform.

When the controller computes a new desired power allocation for a node, the value is written to a control file monitored by the node agent.

The agent then applies the limit using the underlying hardware interface.

This separation between control logic and actuation ensures that the control plane remains lightweight and portable.

4.6 Fault Tolerance

Joule includes several mechanisms to maintain safe operation under partial system failure.

If the controller detects that a node has stopped reporting telemetry or becomes unresponsive, the system places that node into a fail-safe configuration using a conservative power limit.

Similarly, if communication delays cause telemetry to become stale, the controller temporarily reduces allocations to maintain the global power envelope.

These safeguards ensure that system faults cannot cause sustained violations of the power cap.

4.7 Implementation

The current Joule prototype consists of approximately 1,750 lines of code across the controller and node agents, excluding reporting and analysis tooling.

The controller implements the global feedback control loop and marginal power allocation logic, while node agents are responsible for telemetry collection and hardware power-limit enforcement. The system communicates using lightweight periodic telemetry updates and simple control messages.

Despite its small implementation size, the system is capable of maintaining tight tracking of a global power envelope while dynamically reallocating power budgets across nodes.

5. Experimental Methodology

To evaluate Joule, we conducted a series of controlled experiments comparing Joule’s dynamic power management with a static baseline configuration.

5.1 Hardware Platform

Experiments were conducted on a multi-node compute cluster. Each node exposes hardware power monitoring and power-limit interfaces that allow software control of processor power consumption.

5.2 Workloads

The evaluation uses a mixture of compute workloads designed to exercise different resource characteristics. These include CPU-bound tasks, machine-learning workloads, and hashing workloads.

This diversity ensures that the system experiences realistic fluctuations in power consumption and throughput.

5.3 Experimental Procedure

Each experiment consists of two runs performed under identical conditions:

- Baseline run
 Nodes operate under static power limits without dynamic control.

- Joule run
 Joule dynamically adjusts node power limits while enforcing the same global power cap.

Workloads and system configuration remain constant between the two runs.

5.4 Cap Sweep

Experiments are repeated across multiple power-cap levels in order to evaluate Joule’s behaviour under different envelope sizes.

The evaluation includes caps ranging from low to moderate power levels, allowing us to observe how Joule performs as available power increases.

Each experiment is executed for approximately six hours to ensure stable measurements and capture workload variability.

5.5 Metrics

Evaluation focuses on five primary metrics:

1. Cap utilisation

mean(power / cap)

Measures how effectively the system uses the available power envelope.

2. Tracking Error

cap − power

Measures how closely the system follows the configured power limit.

3. Overshoot

Percentage of time where:

power > cap

Overshoot indicates violations of the power envelope.

4. Throughput

Aggregate useful work completed per second.

5. Throughput per Watt

work / power

Measures energy efficiency.

5.6 Control Plane Overhead

Joule is designed to operate as a lightweight control layer with minimal runtime overhead. During sustained workload execution, the controller process consumes negligible system resources relative to the workloads being managed. Monitoring using standard system tools indicates that the controller typically uses less than 1% of a single CPU core and approximately 10 MB of memory.

Node agents exhibit similarly small resource footprints while periodically reporting telemetry and enforcing hardware power limits. These results indicate that Joule can operate continuously without materially affecting workload execution.

5.7 Reproducibility

All experiments described in this work can be reproduced using the open-source Joule implementation. The repository includes scripts for running baseline and Joule experiments, as well as tools for generating the evaluation reports.

6. Results

This section evaluates Joule’s ability to maintain a strict global power envelope while improving utilisation of available power. Experiments were conducted across a sweep of power caps ranging from 20 W to 35 W. Each configuration was executed for approximately twelve hours to ensure stable measurements. Throughput is measured as units of completed work per second, where each unit corresponds to a fixed amount of computational work performed by the workload.

Across power caps ranging from 20 W to 35 W, Joule increases average power-cap utilisation from approximately 80–85% under static configuration to approximately 98% while maintaining zero cap violations and improving throughput by 5–12%. These results indicate that a large portion of unused power headroom in statically capped systems can be safely reclaimed through dynamic control.

Compared with a baseline configuration in which nodes operate under static power limits, Joule consistently maintains tight tracking of the global power envelope while increasing useful compute throughput across all evaluated caps.

6.1 Power Tracking

Joule maintains stable tracking of the configured power envelope using the feedback control mechanism described in Section 3.

Across all experiments, the controller maintained average power within approximately 1 W of the configured cap while preventing overshoot events. Observed power fluctuations remained small, with standard deviation typically below 0.6 W and average power remaining within approximately 0.3-1.0W.

These results indicate that the control loop successfully compensates for workload variability while maintaining stable system behaviour.

6.2 Cap utilisation

Power-cap utilisation measures how effectively the system uses the available power envelope:

utilisation = power / cap

Table 1 summarizes utilisation results across the cap sweep.

| Cap (W) | Baseline utilisation | Joule utilisation |
| --- | --- | --- |
| 20 | 0.836 | 0.987 |
| 25 | 0.788 | 0.990 |
| 30 | 0.788 | 0.979 |
| 35 | 0.864 | 0.972 |

Under static configuration, baseline utilisation ranges from approximately 79% to 86%, reflecting the common operational practice of leaving power headroom to avoid violating power constraints.

Joule increases utilisation to 97–99% of the configured cap across all tested envelopes. This improvement demonstrates that dynamic power allocation can safely reclaim unused power headroom.

6.3 Throughput Improvement

Improved utilisation directly translates into increased computational throughput.

Table 2 summarizes aggregate throughput measurements across the cap sweep.

| Cap (W) | Baseline Throughput | Joule Throughput | Improvement |
| --- | --- | --- | --- |
| 20 | 2549 work/s | 2848 work/s | +11.7% |
| 25 | 2884 work/s | 3245 work/s | +12.5% |
| 30 | 3217 work/s | 3456 work/s | +7.4% |
| 35 | 3485 work/s | 3653 work/s | +4.8% |

Joule improves throughput by approximately 5–12% across the evaluated caps.

The magnitude of improvement decreases as the cap increases. At higher power levels the baseline configuration naturally approaches hardware limits, reducing the amount of reclaimable headroom.

6.4 Cap Error Distribution

Tracking error is defined as:

error = P_cap ​− P_total​

The error distribution remains tightly centred near zero under Joule control.

Histogram analysis shows that most samples fall within approximately ±1 W of the cap, indicating stable envelope tracking despite workload variability.

In contrast, the baseline configuration exhibits consistently larger negative error values, reflecting unused power headroom.

6.5 Overshoot

Across all experiments, Joule maintained strict adherence to the configured power envelope. No samples exceeded the configured cap.

Across all evaluated runs, maximum observed overshoot was 0 W and the fraction of time above the configured cap was 0%.

7. Implications

Improving power-cap utilisation has important implications for modern computing infrastructure, particularly in environments where electrical power rather than processor capacity is the primary limiting factor.

In many environments, power availability - not processor capacity - is the primary limiting factor for compute throughput. Datacentres often operate near facility-level power limits, making it difficult to increase compute capacity without major infrastructure upgrades.

By enabling systems to operate closer to their allowed power envelope, Joule effectively increases the amount of useful work that can be performed within existing power constraints.

This capability improves the efficiency of compute clusters, reduces wasted power headroom, and increases throughput without requiring additional hardware or facility expansion.

8. Limitations

While the experimental results demonstrate the effectiveness of Joule’s power allocation strategy, several limitations should be noted.

First, the current evaluation is conducted on a relatively small cluster. Although the system architecture is designed to scale across larger deployments, the experiments presented here focus on a limited number of nodes in order to provide controlled and reproducible measurements. In larger clusters, additional factors such as network latency, telemetry aggregation overhead, and controller scalability may influence system behaviour. Future evaluation on larger systems will be necessary to fully characterize Joule’s performance at datacentre scale.

Second, the workloads used in the evaluation represent a mixture of CPU-bound and compute-oriented tasks, including hashing and machine-learning workloads. While this workload mix introduces meaningful variation in power consumption and throughput, it does not capture the full diversity of real datacentre workloads. In particular, workloads with highly bursty behaviour or strong I/O dependencies may exhibit different power dynamics. Additional experiments across a broader workload spectrum would help further validate Joule’s generality.

Third, Joule currently estimates marginal throughput benefit using lightweight probing and exponentially weighted moving averages. This approach is computationally inexpensive and adapts well to gradual workload changes, but it may not always capture complex or rapidly shifting performance–power relationships. More sophisticated modeling techniques could potentially improve allocation decisions in highly dynamic environments.

Fourth, the present implementation relies on processor-level power management interfaces for actuation. While these mechanisms are widely available on modern systems, the granularity and responsiveness of hardware power control can vary across platforms. As a result, control precision may depend partly on the characteristics of the underlying hardware.

Finally, the current system focuses primarily on maintaining a global power envelope and improving utilisation within that envelope. It does not yet incorporate higher-level scheduling policies that account for job priorities, service-level objectives, or workload fairness. Integrating Joule with cluster schedulers represents an important direction for future work.

Despite these limitations, the results demonstrate that dynamic power allocation can substantially improve utilisation of power-constrained compute environments.

While experiments were conducted on a small cluster for controlled evaluation, Joule’s control model operates on aggregate cluster power and scales linearly with node count. Larger clusters are expected to exhibit smoother aggregate power dynamics due to workload averaging effects.

9. Related Work

Power management has long been an important topic in computer systems research. Prior work spans processor-level power control, cluster scheduling strategies, and datacentre-scale power management. Joule differs from these approaches by treating power itself as a first-class schedulable resource at the cluster level and dynamically allocating node power budgets using a feedback-driven control loop.

9.1 Processor-Level Power Management

Modern processors provide hardware mechanisms for controlling power consumption, including dynamic voltage and frequency scaling (DVFS) and package-level power limits. Interfaces such as Intel Running Average Power Limit (RAPL) allow software to impose power caps on processor packages at runtime.

These mechanisms are widely used to enforce thermal and power constraints at the processor level. However, they are typically applied statically or through local control policies. Joule builds on these hardware interfaces but applies them as actuators within a cluster-wide control system.

9.2 Datacentre Power Management

Datacentre operators often manage power consumption through a combination of static provisioning, workload scheduling, and facility-level power budgeting. Previous research has explored techniques for coordinating power usage across servers in order to avoid exceeding datacentre power limits.

Many of these approaches rely on workload scheduling decisions to indirectly influence power consumption. By contrast, Joule directly controls node power limits in real time, allowing the system to track a global power envelope while workloads continue executing.

9.3 Cluster Scheduling and Resource Allocation

Traditional cluster schedulers treat compute resources - such as CPUs, GPUs, and memory - as the primary resources to allocate. Power is typically treated as a secondary constraint that limits how many nodes or jobs can run simultaneously.

Joule inverts this relationship by treating power itself as the primary resource. Rather than scheduling jobs to approximate a safe power envelope, Joule dynamically allocates power budgets across nodes while workloads execute.

9.4 Control-Theoretic Resource Management

Feedback control has been applied in several areas of computer systems, including CPU scheduling, thermal management, and datacentre cooling. Control-based approaches are well suited for managing systems with fluctuating workloads and uncertain operating conditions.

Joule adopts this perspective by using a feedback controller to maintain a global power envelope. By continuously measuring system behaviour and adjusting power limits in response, the controller maintains stable operation while adapting to workload variability.

9.5 Summary

Existing work provides mechanisms for controlling processor power and coordinating resources within clusters and datacentres. Joule builds on these foundations but introduces a distinct control-plane abstraction: power is treated as a shared cluster resource that can be dynamically allocated across nodes in order to maximize useful work while maintaining strict compliance with a global power envelope.

10. Future Work

Several avenues exist for extending Joule.

Future work includes evaluating the system on larger clusters, integrating additional workload types, and exploring hierarchical power management across rack- and datacentre-level envelopes.

Another promising direction is integrating Joule with cluster schedulers so that job placement decisions can incorporate power availability alongside traditional resource constraints.

Such integration could enable power-aware scheduling policies that maximize cluster-wide efficiency while respecting facility-level power budgets.

11. Conclusion

This paper introduced Joule, a dynamic power allocation system designed to improve utilisation of power-constrained compute clusters. Joule treats power as a shared resource across the cluster and uses a feedback control loop to continuously redistribute power budgets in response to workload behaviour.

In power-constrained environments, Joule allows operators to recover stranded electrical headroom and convert it into additional compute without changing hardware or increasing the configured power envelope.

By monitoring node-level power consumption and throughput, Joule maintains tight tracking of a global power envelope while allocating available headroom to nodes that can convert additional power into useful work. The system combines centralized control with lightweight node agents and hardware power management interfaces to enable continuous adjustment of power limits without interrupting running workloads.

Experimental evaluation demonstrates that Joule substantially improves power-cap utilisation relative to static power configurations. By dynamically reclaiming unused headroom, the system converts previously wasted power capacity into additional computational throughput while maintaining strict adherence to the configured power cap.

These results suggest that dynamic power allocation can serve as a practical control-plane primitive for modern computing environments where power availability is a primary limiting factor. Rather than treating power caps as rigid static constraints, systems can instead operate near their allowed envelope while adapting continuously to workload variability.

Future work will explore scaling Joule to larger clusters, integrating the system with job schedulers, and extending the control model to hierarchical power management across rack- and datacentre-level power envelopes.